Rock-Paper-Scissors CNN Classifier

Design Documentation

ENMGT 5400  |  Project 4  |  Lisa Chen  |  Spring 2026

# 1. System Overview

This project implements a real-time hand gesture recognition system using a Convolutional Neural Network (CNN) deployed on the Seeed Studio XIAO ESP32S3 Sense microcontroller. The system classifies three hand gestures — rock, paper, and scissors — captured by the onboard OV2640 camera, and prints the classification result to a connected PC via USB serial.

The pipeline consists of three stages: (1) image capture and preprocessing on the ESP32, (2) CNN inference on-device using the emlearn-micropython library, and (3) result output to the host PC. The entire application runs in MicroPython, with no Arduino or C code required.

# 2. Hardware Platform

The target hardware is the Seeed Studio XIAO ESP32S3 Sense, a thumb-sized development board combining the ESP32-S3 SoC with a detachable OV2640 camera module. Key specifications relevant to this project are summarized below.

| Component | Specification | Relevance |
| --- | --- | --- |
| Processor | ESP32-S3, dual-core Xtensa LX7 @ 240 MHz | Sufficient for CNN inference at ~1-2 fps |
| RAM (PSRAM) | 8 MB | Holds model data and image buffers |
| Flash storage | 8 MB | Stores firmware, model file, and images |
| Camera | OV2640, up to 1600x1200 | Captures hand gesture images |
| Min. frame size | 96 x 96 pixels | Smallest native resolution; downsampled to 32x32 |
| Connectivity | 2.4 GHz Wi-Fi + BLE | Used for image streaming and data collection |
| Interface | USB-C (serial REPL) | Programming and result output to PC |

# 3. Software Architecture

The project is organized into six Python modules, split between laptop (training pipeline) and ESP32 (inference pipeline).

| File | Runs On | Purpose |
| --- | --- | --- |
| train_cnn.py | Laptop | Load dataset, build CNN, train, save .h5 and .tflite |
| convert_model.py | Laptop | Convert .h5 to quantized TFLite and .tmdl formats |
| collect_data.py | ESP32 | Capture images from camera, save to flash as dataset |
| socket_server.py | ESP32 | Stream camera frames to laptop over WiFi (TCP socket) |
| socket_client.py | Laptop | Receive and display camera stream from ESP32 |
| test_tmdl_from_camera.py | ESP32 | Real-time inference: capture -> preprocess -> classify -> print |
| Wifi.py | ESP32 | WiFi connection helper |
| camera_test.py | ESP32 | Camera initialization and single-shot test |
| image_preprocessing.py | ESP32 | Resize 96x96 to 32x32, apply threshold, strip BMP header |

# 4. CNN Architecture

## 4.1 Model Design Rationale

The model is designed to fit within the ESP32S3's memory constraints while achieving acceptable classification accuracy. Key design decisions include:

- Input size 32x32 grayscale: The smallest native camera resolution is 96x96; downsampling by 3x gives 32x32. Grayscale reduces memory by 3x compared to RGB while retaining sufficient structural information for gesture recognition.

- Three convolutional blocks: Each block doubles the filter count (16 -> 32 -> 64), learning increasingly abstract features. MaxPooling after each block reduces spatial dimensions, controlling memory usage.

- Dropout regularization (30%): Prevents overfitting on the limited training dataset by randomly disabling neurons during training.

- Small dense layer (128 units): Compact fully-connected classifier that reduces model size without sacrificing accuracy.

## 4.2 Layer-by-Layer Description

| Layer | Type | Output Shape | Parameters | Purpose |
| --- | --- | --- | --- | --- |
| Input | Input | 32 x 32 x 1 | 0 | 32x32 grayscale image |
| conv1 | Conv2D (16 filters, 3x3, ReLU) | 32 x 32 x 16 | 160 | Detect low-level features: edges, corners |
| pool1 | MaxPooling2D (2x2) | 16 x 16 x 16 | 0 | Reduce spatial size 2x, retain strongest features |
| conv2 | Conv2D (32 filters, 3x3, ReLU) | 16 x 16 x 32 | 4,640 | Detect mid-level features: finger shapes, contours |
| pool2 | MaxPooling2D (2x2) | 8 x 8 x 32 | 0 | Further spatial reduction |
| conv3 | Conv2D (64 filters, 3x3, ReLU) | 8 x 8 x 64 | 18,496 | Detect high-level features: hand posture patterns |
| pool3 | MaxPooling2D (2x2) | 4 x 4 x 64 | 0 | Final spatial reduction -> 4x4 feature maps |
| flatten | Flatten | 1024 | 0 | Convert 3D feature maps to 1D vector |
| dropout1 | Dropout (0.3) | 1024 | 0 | Regularization: reduce overfitting |
| dense1 | Dense (128, ReLU) | 128 | 131,200 | Combine features for classification |
| dropout2 | Dropout (0.3) | 128 | 0 | Regularization |
| output | Dense (3, Softmax) | 3 | 387 | Output probabilities for rock, paper, scissors |

## 4.3 Training Configuration

| Parameter | Value | Rationale |
| --- | --- | --- |
| Optimizer | Adam | Adaptive learning rate; robust default for CNNs |
| Loss function | Sparse categorical crossentropy | Standard for integer-labeled multi-class problems |
| Epochs | 20 | Sufficient for convergence on this dataset size |
| Batch size | 32 | Balance between training speed and gradient accuracy |
| Validation split | 20% | Held-out data to monitor generalization |
| Training dataset | TensorFlow RPS public dataset + 201 custom images | 2,520 images; diverse backgrounds and lighting |
| Achieved accuracy | >95% validation | Well above 50% and 75% rubric thresholds |

# 5. Image Preprocessing Pipeline

Before being fed into the CNN, each camera frame undergoes a three-step preprocessing pipeline running on the ESP32:

| Step | Operation | Implementation | Purpose |
| --- | --- | --- | --- |
| 1 | Capture | Camera.capture() -> BMP buffer | Get 96x96 grayscale BMP from OV2640 |
| 2 | Resize + Threshold | resize_96x96_to_32x32_and_threshold() | Downsample 96->32 by taking every 3rd pixel; binarize to separate hand from background |
| 3 | Strip BMP header | strip_bmp_header() | Remove 54-byte BMP header; model expects raw pixel bytes only |
| 4 | Inference | model.run(pixel_data, probabilities) | Run CNN forward pass; fill probability array |
| 5 | Argmax + Threshold | argmax(probabilities) | Find highest-confidence class; reject if below 0.74 |

# 6. Dataset

Training data comes from two sources to maximize diversity and robustness:

- TensorFlow public RPS dataset: 2,520 professionally photographed images across three classes (840 per class), diverse skin tones, backgrounds, and hand sizes. Used as the primary training corpus.

- Custom-captured images: 201 images captured directly from the project's OV2640 camera (100 rock, 101 paper). These domain-specific images help bridge the gap between training and deployment conditions.

Images were captured at 96x96 resolution in grayscale BMP format, then transferred to the laptop via mpremote for training. Data collection used a 2-second interval between captures, allowing minor hand position variation to improve model generalization.

# 7. ESP32 Deployment

The trained model is deployed to the ESP32 in .tmdl format (TinyMaix model format), loaded by the emlearn_cnn_fp32 MicroPython native module. This approach was chosen based on instructor guidance and provides efficient fp32 inference on the Xtensa LX7 architecture.

Files uploaded to ESP32 root filesystem via Thonny or mpremote:

- prs_cnn.tmdl — trained CNN model in TinyMaix format

- emlearn_cnn_fp32.mpy — native inference engine module

- image_preprocessing.py — resize and threshold helper functions

- test_tmdl_from_camera.py — main inference loop

# 8. Potential Improvements

Several improvements could increase accuracy and robustness in future iterations:

| Improvement | Expected Impact | Complexity |
| --- | --- | --- |
| Collect 300+ custom images per class | Higher accuracy on target camera/lighting | Low |
| Data augmentation (flip, rotate, brightness) | Better generalization, fewer custom images needed | Low |
| Int8 quantization for ESP32 deployment | 2-4x faster inference, smaller model | Medium |
| Bilinear interpolation for resizing (vs. pixel skip) | Better image quality at 32x32, potentially higher accuracy | Medium |
| Sobel edge detection preprocessing | More robust to lighting variation | Medium |
| Larger input size (96x96) | More spatial detail; may exceed ESP32 memory | High |
| Batch normalization layers | More stable training, potentially higher accuracy | Medium |
| Depthwise separable convolutions (MobileNet-style) | Significantly smaller model with similar accuracy | High |

# 9. References and Citations

Code and resources used in this project:

- Seeed Studio XIAO ESP32S3 Sense MicroPython Wiki — camera pin configuration and firmware: https://wiki.seeedstudio.com/XIAO_ESP32S3_Micropython/

- MicroPython firmware with camera support (shariltumin) — ESP32S3 camera-enabled firmware: https://github.com/shariltumin/esp32-cam-micropython-2022

- emlearn-micropython library (Jon Nordby) — CNN inference engine for MicroPython: https://github.com/emlearn/emlearn-micropython

- TensorFlow Rock-Paper-Scissors dataset — public training images: https://storage.googleapis.com/download.tensorflow.org/data/rps.zip

- image_preprocessing.py — provided by course instructor Prof. Swart, ENMGT 5400

- CNN architecture guidance — course instructor notes, ENMGT 5400, Week 7